Stackup

https://jlcpcb.com/pcb-impedance-calculator

| Impedance (Ω) | Purpose | Type | Signal Layer | Top Ref | Bottom Ref | Trace Width | Trace Spacing | Impedance trace to copper |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 85 | Top Layer 85ohm… | Differential Pair (Non coplanar) | L1 | / | L2 | 4.08 | 4.2 | / |
| 90 |  | Differential Pair (Non coplanar) | L1 | / | L2 | 4.05 | 5.5 | / |
| 85 |  | Differential Pair (Non coplanar) | L3 | L2 | L5 | 4.03 | 5 | / |
| 50 |  | Single Ended (Non coplanar) | L1 | / | L2 | 4.31 | / | / |
| 48.88 |  | Single Ended (Non coplanar) | L3 | L2 | L5 | 4 | / | / |

| Protocol | Layer | Trace Width | Trace Spacing | Target Single | Target Diff |
| --- | --- | --- | --- | --- | --- |
| MIPI CSI, DSI | L1/L6 | 4.31mil | 6.4mil | 50ohm | 90ohm |
| USB 3.1, USB 2.0, SuperSpeed & PCIe, HDMI, DP, DDR4 Differential | L1/L6 | 4.02mil | 5.4mil | - | 90ohm |
| DDR4, RGMII, SDMMC | L1/L6 | 4.31mil | - | 50ohm | - |
| DDR4, RGMII, SDMMC | L3/L4 | 4.0mil | - | 48.88ohm |  |

| Protocol | Layer | Trace Width | Trace Spacing | Target Single | Target Diff |
| --- | --- | --- | --- | --- | --- |
| 90ohm diff 50ohm single | L1/L6 | 4.31mil | 6.4mil | 50ohm | 90ohm |
| USB 3.1, USB 2.0, SuperSpeed & PCIe, HDMI, DP, DDR4 Differential | L1/L6 | 4.02mil | 5.4mil | - | 90ohm |
| DDR4, RGMII, SDMMC | L1/L6 | 4.31mil | - | 50ohm | - |
| DDR4, RGMII, SDMMC | L3/L4 | 4.0mil | - | 48.88ohm |  |

For non-coplanar keep trace spacing between neighboring traces 3 times the width apart if measuring from the center. This means trace spacing would be equal 2x the trace width.

50ohm single ended non coplanar (top layer):